# Drop-Out Fuse (DOF) / Isolator Switch

1. Introduction:-

Drop-Out Fuses (DOFs), also known as Drop-Out Fuse Isolators, are protective devices used in medium-voltage overhead power distribution systems. They serve a dual purpose: protecting equipment from overcurrent and providing a visible disconnection point for maintenance.

2. Construction and Components:-

| Component | Description |
| --- | --- |
| Fuse Carrier | Holds the fuse link; drops out when the fuse blows, providing visual cue. |
| Fuse Link | A replaceable element that melts under fault current to interrupt the circuit. |
| Insulators | Typically made of porcelain or polymer; support and isolate live parts. |
| Hinge & Latch Assembly | Allows the fuse carrier to pivot and drop when the fuse blows. |
| Contacts | Ensure electrical continuity when the fuse is intact. |

3)Working Principle:-

- Under normal conditions, current flows through the fuse link.

- When a fault occurs (e.g., short circuit or overload), the fuse link melts.

- The fuse carrier drops down due to gravity and hinge design.

- This breaks the circuit and provides a visible disconnection, ensuring safety for maintenance.

- Technical Specifications (Typical for 11 kV DOF):-

| Parameter | Value |
| --- | --- |
| Rated Voltage | 11 kV |
| Rated Current | 100 A to 200 A |
| Breaking Capacity | Up to 10 kA |
| Insulation Type | Porcelain / Polymeric |
| Mounting | Pole-mounted |
| Operating Temperature | -10°C to +55°C |
| Standard Compliance | IS 9385 / IEC 60282-2 |

5. Applications:-

- Protection of distribution transformers

- Isolation of faulty sections in overhead lines

- Tap-off points in rural electrification

- Substation feeder protection

6. Advantages:-

✅ Cost-effective protection
✅ Simple visual indication of fuse operation
✅ Easy to replace fuse links
✅ Minimal maintenance required
✅ Improved safety during maintenance due to visible isolation

7.Diagram:-

8. Safety Considerations:-

- Always isolate upstream supply before replacing fuse links.

- Use insulated tools and PPE.

- Ensure proper earthing of the system.

- Regular inspection for corrosion or mechanical wear.

9. Comparison: DOF vs Isolator Switch:-

| Feature | Drop-Out Fuse (DOF) | Isolator Switch |
| --- | --- | --- |
| Protection | Yes (fuse link melts) | No (manual operation only) |
| Visual Disconnection | Yes (fuse carrier drops) | Yes (manual open/close) |
| Automation | Manual | Manual or motorized |
| Cost | Lower | Higher |
| Maintenance | Low | Moderate |
| Application | Overhead lines, transformers | Substations, switching yards |

10. Conclusion:-

Drop-Out Fuses are essential for reliable and safe operation of medium-voltage distribution networks. Their simplicity, cost-effectiveness, and dual functionality make them a preferred choice in rural and semi-urban power systems.

Presented By:-1)Harshad Gholap

2)Atul Yuvare

Guide By:-Prof.Bhushan Kadam